BAE587 – Homework# 5

The entire code for the homework is in the “BAE587-QClab-code-examples.Rmd”.

Learning objectives:

- Recognize hydrographs and flow patterns during a hydrological event

- Recognize a chemograph and define its descriptors

- Summarize what the arithmetic average and median concentrations are for common pollutants in mostly agricultural watersheds

- Calculate cumulative flow volumes expressed in cubic meters or in mm of flow depth

- Calculate cumulative load of materials and nutrients and express them in kg and kg/ha

- Extract from both cumulative indicators above the flow weighted concentrations

- Recognize the graphical representation of Flow Weighted Concentrations CumulLoad vs. CumulFlow curves

- Distinguish between arithmetic and flow weighted concentrations for a variety of common pollutants in mostly agricultural watersheds

- Recognize the disproportional importance of high flows to the export of pollutants in watersheds

- Write your results in a report style where all the text and codes for equations and figures are embedded into one document using R Markdown or Quarto.

The goal of this homework is for you to manipulate and report some very useful tools to derive important hydrology and water quality indicators in watersheds. For this, I am asking you to apply the concept of reproducible research where all the information to write your report is embedded into one single document (i.e., no text written using one software and figures copied into it from another one). I propose that you use R Markdown or Quarto codes in R Studio to write and illustrate your homework/report.

All instructions and documents are stored on the Github repository: https://github.com/francoisbirgand/QClab587. Go to this link. Click on the green “<> code” button and choose ‘download zip’. Unzip the file and place all the unzip files in a specific directory (e.g., BAE587-hmk5).

Open and read the file “BAE587-QCHmk5-usingR.pptx”. Many of the technical instructions are there to use RStudio. If you know nothing about R studio and R, follow all instructions to download them. You may totally work in Python if you prefer and do the homework with it. In all cases, make sure you use the reproducible research platform.

In the document entitled “BAE587-QC-lecture-and-code.Rmd” written in R Markdown, I give a quick lecture on what hydrographs are, and some tricks to calculate cumulative flow and loads. Borrow all the code you need to answer the homework questions.

The study watershed for this homework is the Chickasaw River at Franklin, Ohio (lat:40.487038, long:-84.481683). The watershed is 42.5 square kilometers in surface area, draining a predominantly flat, tile drained, agricultural watershed.

Exercise 1 (10 points):

Present and discuss your observations on the yearly 2012-2013 hydrograph for the Chickasaw River at Franklin, Ohio.

You may borrow and modify the code available given in “BAE587-QC-lecture-and-code.Rmd” to answer the question.

For the discussion, think about seasonality (or lack thereof) of baseflow, of the relative magnitude of flow peaks compared to base flow just before peaks. Propose possible reasons for your observations (hint: the description of the watershed should help you).

Exercise 2 (20 points):

Calculate and report the lowest, highest, mean, and median flow rates, as well as the 10th percentile, and the 90th percentile. For this, there is the quantile function, which works as quantile(Q,percentile value).

Report them in a bullet list (you may read this crash course on R markdown to help you for the synthax).

Exercise 3 (20 points):

Calculate and report the arithmetic average and median concentrations for nitrate, TP, SRP, and TSS over the entire year of record.

List reference concentration values for agricultural watersheds seen in lecture and compare your results to these reference values.

Exercise 4 (20 points):

Plot on the same graph the hydrograph and the corresponding chemograph for each of the water quality parameter nitrate, TP, SRP, and TSS (four plots, one per parameter) for the Chickasaw River at Franklin, Ohio, from 2013-02-25 12:00 to 2013-03-05 12:00.

You may use code available at https://francoisbirgand.github.io/BAE-587/introduction-to-hydrographs-chemographs-concentrations-and-loads.html#chemographs-and-concentration-levels

For each, report whether the chemographs exhibited a concentration or a dilution effect.

Exercise 5 (15 points):

Over the same 10-day period, for each of the nitrate, TP, SRP, and TSS, plot the cumulative load expressed in kg/km²/10 days as a function of the cumulative flow volume expressed in mm. Add a line connecting the first to the last point of the curve.

A code to calculate cumulative volume or load is given in the same book section given above.

Report in the text below the arithmetic average and the EMC concentrations, and calculate and report their percentage difference. You may use embedded code to make all your calculations and display automatically in the text. Discuss the differences between the two concentration values and provide explanations for the differences.

Exercise 6 (15 points):

Over the entire 2012-2013 hydrological year, for each of the nitrate, TP, SRP, and TSS, plot the cumulative load expressed in kg/km²/year as a function of the cumulative volume expressed in mm. Add a line connecting the first to the last point of the curve.

Report in the text below the plots the arithmetic average and the EMC concentrations, and calculate and report their percentage difference. You may use embedded code to make all your calculations and display in the text. Discuss and provide explanations the differences between the two. Discuss the differences between your findings at the 10-day event and the annual scales.